Wireshark Lab: Ethernet and ARP

Version: 2.0

© 2007 J.F. Kurose, K.W. Ross. All Rights Reserved

Computer Networking: A Top- down Approach, 4th edition.

In this lab, we’ll investigate the Ethernet protocol and the ARP protocol. Before beginning this lab, you’ll probably want to review sections 5.5 (Ethernet), 5.4.1 (link- layer addressing) and 5.4.2 (ARP) in the text. RFC 826 (ftp://ftp.rfc-editor.org/in- notes/std/std37.txt) contains the gory details of the ARP protocol, which is used by an IP device to determine the IP address of a remote interface whose Ethernet address is known.

# Capturing and analyzing Ethernet frames

Let’s begin by capturing a set of Ethernet frames to study.  Do the following1:

  - First, make sure your browser’s cache is empty. (To do this under Netscape 7.0, select Edit->Preferences->Advanced->Cache and clear the memory and disk cache. For Internet Explorer, select Tools->Internet Options->Delete Files. For Firefox select Tools->Clear Private Data.

  - Start up the Wireshark packet sniffer

  - Enter the following URL into your browser http://gaia.cs.umass.edu/wireshark-labs/HTTP-ethereal-lab-file3.html Your browser should display the rather lengthy US Bill of Rights.

- If you are unable to run Wireshark live on a computer, you can download the zip file http://gaia.cs.umass.edu/wireshark-labs/wireshark-traces.zip and extract the file ethernet--ethereal-trace-1. The traces in this zip file were collected by Wireshark running on one of the author’s computers, while performing the steps indicated in the Wireshark lab. Once you have downloaded the trace, you can load it into Wireshark and view the trace using the File pull down menu, choosing Open, and then selecting the ethernet-ethereal-trace-1 trace file. You can then use this trace file to answer the questions below.

  - Stop Wireshark packet capture. First, find the packet numbers (the leftmost column in the upper Wireshark window) of the HTTP GET message that was sent from your computer to gaia.cs.umass.edu, as well as the beginning of the HTTP response message sent to your computer by gaia.cs.umass.edu. You should see a screen that looks something like this (where packet 4 in the screen shot below contains the HTTP GET message)

  - Since this lab is about Ethernet and ARP, we’re not interested in IP or higher- layer protocols. So let’s change Wireshark’s “listing of captured packets” window so that it shows information only about protocols below IP. To have Wireshark do this, select Analyze->Enabled Protocols. Then uncheck the IP box and select OK. You should now see an Wireshark window that looks like:

In order to answer the following questions, you’ll need to look into the packet details and packet contents windows (the middle and lower display windows in Wireshark).

Select the Ethernet frame containing the HTTP GET message. (Recall that the HTTP GET message is carried inside of a TCP segment, which is carried inside of an IP datagram, which is carried inside of an Ethernet frame; reread section 1.7.2 in the text if you find this nesting a bit confusing). Expand the Ethernet II information in the packet details window. Note that the contents of the Ethernet frame (header as well as payload) are displayed in the packet contents window.

Answer the following questions, based on the contents of the Ethernet frame containing the HTTP GET message. Whenever possible, when answering a question you should hand in a printout of the packet(s) within the trace that you used to answer the question asked. Annotate the printout to explain your answer. To print a packet, use File->Print, choose Selected packet only, choose Packet summary line, and select the minimum amount of packet detail that you need to answer the question.

- What is the 48-bit Ethernet address of your computer?

- What is the 48-bit destination address in the Ethernet frame? Is this the Ethernet address of gaia.cs.umass.edu.

- How large is the ARP header for a request? What about for a reply?

- Give the hexadecimal value for the two-byte Frame type field. What do the bit(s) whose value is 1 mean within the flag field?

- How many bytes from the very start of the Ethernet frame does the ASCII “G” in “GET” appear in the Ethernet frame?

- What is the hexadecimal value of the CRC field in this Ethernet frame?

- Is the ARP reply broadcast (like the ARP request) or not?

- What Ethernet Type value which indicates that ARP is the higher layer protocol?

Next, answer the following questions, based on the contents of the Ethernet frame containing the first byte of the HTTP response message.

- What is the value of the Ethernet source address? Is this the address of your computer, or of gaia.cs.umass.edu (Hint: the answer is no). What device has this as its Ethernet address?

- What is the destination address in the Ethernet frame? Is this the Ethernet address of your computer?

- Give the hexadecimal value for the two-byte Frame type field. What do the bit(s) whose value is 1 mean within the flag field?

- How many bytes from the very start of the Ethernet frame does the ASCII “O” in “OK” (i.e., the HTTP response code) appear in the Ethernet frame?

- What is the hexadecimal value of the CRC field in this Ethernet frame?

# The Address Resolution Protocol

In this section, we’ll observe the ARP protocol in action. We strongly recommend that you re-read section 5.4.2 in the text before proceeding.

## ARP Caching

Recall that the ARP protocol typically maintains a cache of IP-to-Ethernet address translation pairs on your comnputer  The arp command (in both MSDOS and Linux/Unix) is used to view and manipulate the contents of this cache. Since the arp command and the ARP protocol have the same name, it’s understandably easy to confuse them. But keep in mind that they are different - the arp command is used to view and manipulate the ARP cache contents, while the ARP protocol defines the format and meaning of the messages sent and received, and defines the actions taken on message transmission and receipt.

Let’s take a look at the contents of the ARP cache on your computer:

  - MS-DOS. The arp command is in c:\windows\system32, so type either “arp” or “c:\windows\system32\arp” in the MS-DOS command line (without quotation marks).

  - Linux/Unix. The executable for the arp command can be in various places. Popular locations are /sbin/arp (for linux) and /usr/etc/arp (for some Unix variants).

The arp command with no arguments will display the contents of the ARP cache on your computer.  Run the arp command.

- Write down the contents of your computer’s ARP cache. What is the meaning of each column value?

In order to observe your computer sending and receiving ARP messages, we’ll need to clear the ARP cache, since otherwise your computer is likely to find a needed IP-Ethernet address translation pair in its cache and consequently not need to send out an ARP message.

  - MS-DOS. The MS-DOS arp –d * command will clear your ARP cache. The –d flag indicates a deletion operation, and the * is the wildcard that says to delete all table entries.

  - Linux/Unix. The arp –d * will clear your ARP cache. In order to run this command you’ll need root privileges. If you don’t have root privileges and can’t run Wireshark on a Windows machine, you can skip the trace collection part of this lab and just use the trace discussed in footnote 1.

## Observing ARP in action

Do the following:

  - Clear your ARP cache, as described above.

  - Next, make sure your browser’s cache is empty. (To do this under Netscape 7.0, select Edit->Preferences->Advanced->Cache and clear the memory and disk cache. For Internet Explorer, select Tools->Internet Options->Delete Files.)

  - Start up the Wireshark packet sniffer

  - Enter the following URL into your browser http://gaia.cs.umass.edu/wireshark-labs/ HTTP-wireshark-lab-file3.html Your browser should again display the rather lengthy US Bill of Rights.

  - Stop Wireshark packet capture. Again, we’re not interested in IP or higher-layer protocols, so change Wireshark’s “listing of captured packets” window so that it shows information only about protocols below IP. To have Wireshark do this, select Analyze->Enabled Protocols. Then uncheck the IP box and select OK. You should now see an Wireshark window that looks like:

- The ethernet-ethereal-trace-1 trace file in http://gaia.cs.umass.edu/wireshark-labs/wireshark-traces.zip was created using the steps below (in particular after the ARP cache had been flushed).

In the example above, the first two frames in the trace contain ARP messages (as does the 6th message).  The screen shot above corresponds to the trace referenced in footnote 1.

Answer the following questions:

- What are the hexadecimal values for the source and destination addresses in the Ethernet frame containing the ARP request message?

- Give the hexadecimal value for the two-byte Ethernet Frame type field. What do the bit(s) whose value is 1 mean within the flag field?

- Download the ARP specification from ftp://ftp.rfc-editor.org/in- notes/std/std37.txt. A readable, detailed discussion of ARP is also at http://www.erg.abdn.ac.uk/users/gorry/course/inet-pages/arp.html.

  - How many bytes from the very beginning of the Ethernet frame does the ARP opcode field begin?

  - What is the value of the opcode field within the ARP-payload part of the Ethernet frame in which an ARP request is made?

  - Does the ARP message contain the IP address of the sender?

  - Where in the ARP request does the “question” appear – the Ethernet address of the machine whose corresponding IP address is being queried?

- Now find the ARP reply that was sent in response to the ARP request.

  - How many bytes from the very beginning of the Ethernet frame does the ARP opcode field begin?

  - What is the value of the opcode field within the ARP-payload part of the Ethernet frame in which an ARP response is made?

  - Where in the ARP message does the “answer” to the earlier ARP request appear – the IP address of the machine having the Ethernet address whose corresponding IP address is being queried?

- What are the hexadecimal values for the source and destination addresses in the Ethernet frame containing the ARP reply message?

- Open the ethernet-ethereal-trace-1 trace file in http://gaia.cs.umass.edu/wireshark-labs/wireshark-traces.zip. The first and second ARP packets in this trace correspond to an ARP request sent by the computer running Wireshark, and the ARP reply sent to the computer running Wireshark by the computer with the ARP-requested Ethernet address.  But there is yet another

computer on this network, as indiated by packet 6 – another ARP request. Why is there no ARP reply (sent in response to the ARP request in packet 6) in the packet trace?

Analyzing ARP on GNS3:

Note: Use the same topology used for ping.

Do the following:

- Ping from PC-3 to PC-2

- Start Wireshark trace on link between PC3 and Switch 3.

- Analyze the packets for ARP.

- Provide screenshots as shown below.

Answer the following question.

- Is ARP request Broadcast or Unicast?

- What is the Ethernet Source address for ARP request?

- What is the Ethernet Destination address for ARP request?

- What is the Ethernet Source address for ARP reply?

- What is the Ethernet Destination address for ARP reply?

- Lab Report

Write a minimum of two type-written pages with a minimum of 12-point font describing what you learned in the lab. Do NOT simply repeat the steps you performed, summarize the purpose of the lab by explaining the concepts covered.

Your lab report will be submitted automatically to turnitn.com (Links to an external site.) when you submit it to Canvas. You do not need to do anything besides submit the assignment to Canvas. Be sure you write your report in your own words. If you don’t I will know it and you will receive zero points for the lab and face possible other consequences. Although you may work with other to compete the lab procedure, your report MUST be your original work.