Notes:

# Capturing and analyzing Ethernet frames

Let’s begin by capturing a set of Ethernet frames to study.  Do the following1:

  - First, make sure your browser’s cache is empty. (To do this under Netscape 7.0, select Edit->Preferences->Advanced->Cache and clear the memory and disk cache. For Internet Explorer, select Tools->Internet Options->Delete Files. For Firefox select Tools->Clear Private Data.

  - Start up the Wireshark packet sniffer

  - Enter the following URL into your browser http://gaia.cs.umass.edu/wireshark-labs/HTTP-ethereal-lab-file3.html Your browser should display the rather lengthy US Bill of Rights.

  - Stop Wireshark packet capture. First, find the packet numbers (the leftmost column in the upper Wireshark window) of the HTTP GET message that was sent from your computer to gaia.cs.umass.edu, as well as the beginning of the HTTP response message sent to your computer by gaia.cs.umass.edu. You should see a screen that looks something like this (where packet 4 in the screen shot below contains the HTTP GET message)

  - Since this lab is about Ethernet and ARP, we’re not interested in IP or higher- layer protocols. So let’s change Wireshark’s “listing of captured packets” window so that it shows information only about protocols below IP. To have Wireshark do this, select Analyze->Enabled Protocols. Then uncheck the IP box and select OK. You should now see an Wireshark window that looks like:

In order to answer the following questions, you’ll need to look into the packet details and packet contents windows (the middle and lower display windows in Wireshark).

Select the Ethernet frame containing the HTTP GET message. (Recall that the HTTP GET message is carried inside of a TCP segment, which is carried inside of an IP datagram, which is carried inside of an Ethernet frame; reread section 1.7.2 in the text if you find this nesting a bit confusing). Expand the Ethernet II information in the packet details window. Note that the contents of the Ethernet frame (header as well as payload) are displayed in the packet contents window.

Answer the following questions, based on the contents of the Ethernet frame containing the HTTP GET message. Whenever possible, when answering a question you should hand in a printout of the packet(s) within the trace that you used to answer the question asked. Annotate the printout to explain your answer. To print a packet, use File->Print, choose Selected packet only, choose Packet summary line, and select the minimum amount of packet detail that you need to answer the question.

- What is the 48-bit Ethernet address of your computer?

- What is the 48-bit destination address in the Ethernet frame? Is this the Ethernet address of gaia.cs.umass.edu.

- How large is the ARP header for a request? What about for a reply?

- Give the hexadecimal value for the two-byte Frame type field. What do the bit(s) whose value is 1 mean within the flag field?

- How many bytes from the very start of the Ethernet frame does the ASCII “G” in “GET” appear in the Ethernet frame?

- What is the hexadecimal value of the CRC field in this Ethernet frame?

- Is the ARP reply broadcast (like the ARP request) or not?

- What Ethernet Type value which indicates that ARP is the higher layer protocol?

Next, answer the following questions, based on the contents of the Ethernet frame containing the first byte of the HTTP response message.

- What is the value of the Ethernet source address? Is this the address of your computer, or of gaia.cs.umass.edu (Hint: the answer is no). What device has this as its Ethernet address?

- What is the destination address in the Ethernet frame? Is this the Ethernet address of your computer?

- Give the hexadecimal value for the two-byte Frame type field. What do the bit(s) whose value is 1 mean within the flag field?

- How many bytes from the very start of the Ethernet frame does the ASCII “O” in “OK”  (i.e., the HTTP response code) appear in the Ethernet frame?

- What is the hexadecimal value of the CRC field in this Ethernet frame?

- What are the hexadecimal values for the source and destination addresses in the Ethernet frame containing the ARP request message?

- Give the hexadecimal value for the two-byte Ethernet Frame type field. What do the bit(s) whose value is 1 mean within the flag field?

- Download the ARP specification from ftp://ftp.rfc-editor.org/in- notes/std/std37.txt. A readable, detailed discussion of ARP is also at http://www.erg.abdn.ac.uk/users/gorry/course/inet-pages/arp.html.

  - How many bytes from the very beginning of the Ethernet frame does the ARP opcode field begin?

  - What is the value of the opcode field within the ARP-payload part of the Ethernet frame in which an ARP request is made?

  - Does the ARP message contain the IP address of the sender?

  - Where in the ARP request does the “question” appear – the Ethernet address of the machine whose corresponding IP address is being queried?

- Now find the ARP reply that was sent in response to the ARP request.

  - How many bytes from the very beginning of the Ethernet frame does the ARP opcode field begin?

  - What is the value of the opcode field within the ARP-payload part of the Ethernet frame in which an ARP response is made?

  - Where in the ARP message does the “answer” to the earlier ARP request appear – the IP address of the machine having the Ethernet address whose corresponding IP address is being queried?

- What are the hexadecimal values for the source and destination addresses in the Ethernet frame containing the ARP reply message?

- Open the ethernet-ethereal-trace-1 trace file in http://gaia.cs.umass.edu/wireshark-labs/wireshark-traces.zip. The first and second ARP packets in this trace correspond to an ARP request sent by the computer running Wireshark, and the ARP reply sent to the computer running Wireshark by the computer with the ARP-requested Ethernet address.  But there is yet another

computer on this network, as indiated by packet 6 – another ARP request. Why is there no ARP reply (sent in response to the ARP request in packet 6) in the packet trace?

- Is ARP request Broadcast or Unicast?

- What is the Ethernet Source address for ARP request?

- What is the Ethernet Destination address for ARP request?

- What is the Ethernet Source address for ARP reply?

- What is the Ethernet Destination address for ARP reply?

- Lab Report