Subnetting

- IP address: 172.121.10.0/27

Answer the following questions

- What is the block size and number of hosts per subnets?

  - This address borrows three bits from the 4th octet, leaving 5 bits for subnetting.

    - 11111111.11111111.11111111.11100000

    - The last octet: (128+64+32)(16+8+4+2+1)

    - Network = 224 | Hosts = 31

  - If we choose to use all 5 bits for a single network then the following is the resultant network: (One network with 32 hosts, 30 usable.)

    - Network address: 172.121.10.0 /27

    - First usable IP address: 172.121.10.1 / 27

    - Last usable IP address: 172.121.10.30 / 27

    - The broadcast address is: 172.121.10.31 / 27

  - We can make smaller networks if needed with as few as 2 usable IP addresses per network.

- IP Address: 192.168.10.200  255.255.255.192

Answer the following questions

- Which subnet does the host belong to?

  - 192.168.10.192 /26 is the network

- What is the first and the last valid host on the subnetwork that the node 192.168.10.200 belong to?

  - First usable IP address is: 192.168.10.193 / 26

  - Last usable IP address is: 192.168.10.254 / 26

- IP Address: 172.26.129.192/28

Answer the following questions

- What is the block size and number of hosts per subnet?

  - 11111111.11111111.11111111.11110000

  - 15 IP addresses on a single subnet (13 usable)

- What is the network address and broadcast address?

  - The network address: 172.26.129.192 / 28

  - The broadcast address: 172.26.129.207 / 28

- What is the first and last valid host?

  - The first usable IP: 172.26.129.193 / 28

  - The last usable IP: 172.26.129.206 / 28

- IP Address: 171.128.80.25/20

Answer the following questions

- How many subnets and hosts per subnet can you get from the above given host?

  - Choice 1: 1 network with 4094 usable hosts

  - Choice 2: 2 networks with 2046 usable hosts per network

  - Choice 3: 4 networks with 1022 usable hosts per network

  - Choice 4: 8 networks with 510 usable hosts per network

  - Choice 5: 16 networks with 254 usable hosts per network

- What is the network address and broadcast address?

  - Using choice 1: Network address: 171.128.80.0 /20

  - Using choice 1: Broadcast address: 171.128.95.255 /20

- What is the valid host range of the subnetwork the given host belong to?

  - Usable hosts for choice 1: 171.128.80.1 /20  171.128.95.254 /20

- IP Address: 10.67.192.0  255.255.252.0

Answer the following questions

- What is the block size of the network?

  - One network with 1022 usable hosts, (1024 total)

- What is the network address and broadcast address of the above given network?

  - Network Address: 10.67.192.0 /22

  - Broadcast Address: 10.67.195.255 /22

- What is the first and the last valid host of the network given?

  - First usable IP address: 10.67.192.1 /22

  - Last usable IP address: 10.67.195.254 /22

- IP address: 129.151.150.21/19

Answer the following questions

- How many subnets and hosts per subnet can you get from the above given host?

  - One subnet with 8192 hosts or as many as 2048 subnets with 2 usable hosts per subnet.

- Which subnet does the host belong to?

  - Assuming a single network: 129.151.128.0 /19

- What is the first and the last valid host?

  - Assuming a single network: First usable IP: 129.151.128.1 / 19

  - Assuming a single network: Last usable IP: 129.151.159.254 /19

- You are designing a subnet mask for the 172.24.0.0 network. You want 50 subnets with up to 800 hosts on each subnet. What subnet mask should you use?

      - A class C would only yield 254 usable hosts. It will require 4 combined class C subnets to yield up to 800 hosts. (4 class C subnets will yield 1022 Usable IP Addresses, 3 class C subnets would not achieve the target of up to 800 hosts.

      - The 172.24.0.0 would need to be a /16 network to be able to yield at least 50 subnets of 1024 hosts.

      - 172.24.0.0 /16 would be subnetted to 64 subnets of 1024 hosts per subnet with each resultant subnet being a /22.

- IP address: 10.200.28.121/12

Answer the following questions

- What is the network address and the broadcast address?

  - The network address is: 10.192.0.0 /12

  - The broadcast address is: 10.207.255.255 /12

- What is the first and the last valid host?

  - The first usable IP: 10.192.0.1 /12

  - The last usable IP: 10.207.255.254 /12

- Which subnet does the host 10.121.32.43/10 belong to and what is the first and last valid host of that network?

      - 10.64.0.0 /10

      - First usable IP: 10.64.0.1 /10

      - Last usable IP: 10.127.255.254 /10

- Consider 3 subnets (Subnet A, Subnet B and Subnet C). Subnet A has 28 hosts, Subnet B has 50 hosts and subnet C has 100 hosts. IP Address given is 192.168.21.32/28. What will be the network address and Broadcast address for Subnet A, B and C. What are is the block size and number of valid hosts per subnet for A, B and C?

    - A:28 hosts | Network: 192.168.21.32 /27; Broadcast: 192.168.21.63/27

    - B:50 hosts | Network: 192.168.21.0 /26; Broadcast: 192.168.21.63/26

    - C:100 hosts| Network: 192.168.21.0 /25; Broadcast: 192.168.21.127/25